《单片机原理与实践》

实验报告书

第四次实验

实验题目：串口通信实验

姓    名：         潘家航

学    号：      202205240220

指导老师：         禹鑫燚

专业班级：      自动化2304

实验地点：        信息楼B114

实验日期：   2025    年  11 月  18 日

一、实验目的

1、了解STM32F407/GD32F407的USART的使用及其相关的API函数；

2、掌握用时间间隔接收串口数据的方法。

二、实验内容

1. 基础实验部分

串口实验

2. 提高实验部分

基于WIFI的TCP服务端实验

基于WIFI的TCP客户端实验

GPS北斗实验

三、实验步骤

1、编程配置STM32F407/GD32F407的USART3，采用中断的方式实现数据的自发白收。

2、使用WIFI模块连接实验板和外部设备实现通信。在基础实验的步骤上，再进行WIFI模块的初始化和设置：首先开启AP模式，然后设置WIFI服务端的名称、密码端口号，最后编写WIFI接发函数。

四、实验主要功能代码注释、分析和实验结果

1. 基础实验部分

（1）实验主要功能代码注释和分析

/****************************************Copyright (c)***************************************

**-------------------------------------------------------------------------------------------

** Created by:          qinyx

** Last modified Date:  2014-02-28

** Last Version:        V1.00

** Descriptions:        STM32F407嵌入式实验箱

**-------------------------------------------------------------------------------------------*/

//********************************************************************************************/

#include "stm32f4xx.h"

#include "USART.h"

#include "delay.h"

#include "LED.h"

int main(void)

{

SysTick_Init();                                                     //  系统滴答定时器初始化

UART3_Configuration();                                              //  USART3配置

printf("USART3 test\r\n");

printf("Input data:\r\n");

while (1)

{

if (Uart3.ReceiveFinish)

{

USART3_Senddata(Uart3.Rxbuf, Uart3.RXlenth);

Uart3.ReceiveFinish = 0;

Uart3.RXlenth = 0;

printf("\r\n");

}

}

}

（2）实验结果和分析

图2 基础实验结果图

2. 提高实验部分

基于WIFI的TCP服务端实验:

/****************************************Copyright (c)***************************************

**-------------------------------------------------------------------------------------------

** Created by:          qinyx

** Last modified Date:  2014-02-28

** Last Version:        V1.00

** Descriptions:        STM32F407嵌入式实验箱

**-------------------------------------------------------------------------------------------*/

//********************************************************************************************/

#include "stm32f4xx.h"

#include "USART3.h"

#include "delay.h"

#include "stdio.h"

#include "KEY.h"

#include "mp3.h"

#include "LED.h"

#include "WIFI.h"

#include "lcd.h"

int main(void)

{

unsigned char temp = 0;

unsigned char test[11] = "WIFI test\r\n";

SysTick_Init();                                                     //  系统滴答定时器初始化

LEDGpio_Init();

KEYGpio_Init();

UART3_Configuration();                                              //  USART3配置

UART5_Configuration(115200);

LCD_Init();                                                         //  液晶初始化

LCD_Clear(WHITE);                                                   //  设置液晶背景

LCD_ShowString(150, 50, "基于WIFI的TCP服务器实验", BLACK, WHITE);

LCD_ShowString(100, 100, "正在初始化wifi模块...", BLACK, WHITE);

printf("正在初始化wifi模块...\r\n");

delay_ms(500);

WIFI_Configuration();               //  wifi配置

LCD_ShowString(100, 100, "wifi模块初始化成功   ", BLACK, WHITE);

printf("wifi模块初始化成功   \r\n");

Uart5.ReceiveFinish = 0;

Uart5.RXlenth = 0;

Uart5.Time = 0;

while (1)

{

temp = KeyScan();

if (temp == 1)

{

WIFI_Send(test, sizeof(test));//  wifi发送数据

}

WIFI_Receive();                 //  wifi接收数据

}

}

基于WIFI的TCP客户端实验:

/****************************************Copyright (c)***************************************

**-------------------------------------------------------------------------------------------

** Created by:          qinyx

** Last modified Date:  2014-02-28

** Last Version:        V1.00

** Descriptions:        STM32F407嵌入式实验箱

**-------------------------------------------------------------------------------------------*/

//********************************************************************************************/

#include "stm32f4xx.h"

#include "USART3.h"

#include "delay.h"

#include "stdio.h"

#include "KEY.h"

#include "mp3.h"

#include "LED.h"

#include "WIFI.h"

#include "lcd.h"

int main(void)

{

unsigned char temp = 0;

unsigned char ON[2] = "ON";

unsigned char OFF[3] = "OFF";

SysTick_Init();                                                     //  系统滴答定时器初始化

LEDGpio_Init();

KEYGpio_Init();

UART3_Configuration();                                              //  USART3配置

//    UART6_Configuration(9600);

UART5_Configuration(115200);

LCD_Init();                                                         //  液晶初始化

LCD_Clear(WHITE);                                                   //  设置液晶背景

LCD_ShowString(150, 50, "基于WIFI的TCP客户端实验", BLACK, WHITE);

LCD_ShowString(100, 80, "模块初始化失败，尝试给模块重新上电", BLACK, WHITE);

LCD_ShowString(100, 100, "正在初始化wifi模块...", BLACK, WHITE);

printf("正在初始化wifi模块...\r\n");

delay_ms(500);

WIFI_Configuration();               //  wifi配置

LCD_ShowString(100, 100, "wifi模块初始化成功   ", BLACK, WHITE);

printf("wifi模块初始化成功   \r\n");

Uart5.ReceiveFinish = 0;

Uart5.RXlenth = 0;

Uart5.Time = 0;

while (1)

{

temp = KeyScan();

if (temp == 1)

{

WIFI_Send(ON, sizeof(ON));

}

else if (temp == 2)

{

WIFI_Send(OFF, sizeof(OFF));

}

WIFI_Receive();

}

}

GPS北斗实验:

/****************************************Copyright (c)***************************************

**-------------------------------------------------------------------------------------------

** Created by:          qinyx

** Last modified Date:  2014-02-28

** Last Version:        V1.00

** Descriptions:        STM32F407嵌入式实验箱

**-------------------------------------------------------------------------------------------*/

//********************************************************************************************/

#include "stm32f4xx.h"

#include "USART3.h"

#include "delay.h"

#include "stdio.h"

#include "KEY.h"

#include "mp3.h"

#include "LED.h"

#include "WIFI.h"

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <stdbool.h>

#include <math.h>

#include "GPS.h"

#include "lcd.h"

int main(void)

{

char dispbuf[100] = "";

SysTick_Init();                                                     //  系统滴答定时器初始化

LEDGpio_Init();

KEYGpio_Init();

UART3_Configuration();                                              //  USART3配置

GPS_Buff_Init();

UART5_Configuration(9600);

printf("GPS+北斗实验\r\n");

LCD_Init();                     //  液晶初始化

LCD_Clear(WHITE);               //  设置液晶背景

LCD_ShowString(180, 50, "GPS+北斗实验", BLACK, WHITE);

while (1)

{

if(RxDataOutPtr != RxDataInPtr)   //if成立的话，说明接收缓冲区有数据了

{

if (memcmp(&RxDataOutPtr[2], "$GNGGA", 6) == 0)

{

GNGGA_info(&RxDataOutPtr[2]);

printf("时间：%s\r\n", gps_time);

printf("经度：%6.2f%c\r\n", longitude,longitude_dir);

printf("纬度：%6.2f%c\r\n", latitude,latitude_dir);

sprintf(dispbuf, "时间：%s", gps_time);

LCD_ShowString(150, 200, (u8*)dispbuf,BLACK, WHITE);

sprintf(dispbuf, "经度：%6.2f%c", longitude,longitude_dir);

LCD_ShowString(150, 220, (u8*)dispbuf,BLACK, WHITE);

sprintf(dispbuf, "纬度：%6.2f%c", latitude,latitude_dir);

LCD_ShowString(150, 240, (u8*)dispbuf,BLACK, WHITE);

}

else if (memcmp(&RxDataOutPtr[2], "$GPGSV", 6) == 0)

{

GSV_info(&RxDataOutPtr[2]);

printf("可见GPS卫星总数:%d\r\n ",gpgsv.numSv);

sprintf(dispbuf, "可见GPS卫星总数:%d\r\n ",gpgsv.numSv);

LCD_ShowString(150, 300, (u8*)dispbuf,BLACK, WHITE);

}

else if (memcmp(&RxDataOutPtr[2], "$BDGSV", 6) == 0)

{

GSV_info(&RxDataOutPtr[2]);

printf("可见北斗卫星总数:%d\r\n ",gpgsv.numSv);

sprintf(dispbuf, "可见北斗卫星总数:%d ",gpgsv.numSv);

LCD_ShowString(150, 320, (u8*)dispbuf,BLACK, WHITE);

}

RxDataOutPtr += RBUFF_UNIT;                       //指针下移

if(RxDataOutPtr == RxDataEndPtr)                  //如果指针到缓冲区尾部了

RxDataOutPtr = RxDataBuf[0];                  //指针归位到缓冲区开头

}

}

}

（2）实验结果和分析

图3 提高实验结果图

# 五、实验中遇到的问题及其解决方法

- 串口无法正常接收数据的问题
问题现象： 实验初期，串口调试助手发送数据后，STM32 并未进入接收中断，Uart3.ReceiveFinish 始终保持为 0。
原因分析： USART3 的波特率设置与上位机不一致，或串口引脚未正确复用为 AF7。
解决方法：

  - 检查 USART3 配置，确认波特率统一设置为 115200；

  - 查看 GPIO 初始化函数，确保 TX（PC10）、RX（PC11）设置了正确的复用模式；

  - 重新烧录程序后，串口通信恢复正常。

- WIFI 模块初始化失败、无法进入 AP 模式
问题现象： LCD 显示“模块初始化失败，尝试给模块重新上电”，串口打印无法收到 WIFI 模块的 AT 指令响应。
原因分析：

  - WIFI 模块供电电压不稳定；

  - UART5 波特率设置不正确；

  - WIFI 模块上电后未预留足够启动时间。
解决方法：

  - 检查 WIFI 模块的 5V 供电插接，确保供电正常；

  - 将 UART5 波特率修改为 115200 与模块一致；

  - 在初始化前增加 500 ms 延迟，模块即可正常进入 AP 模式。

- TCP 客户端无法连接服务器的问题
问题现象： 程序能初始化 WIFI，但无法建立 TCP 连接，按键发送“ON”“OFF”无响应。
原因分析：

  - PC 或手机与 WIFI 模块不在同一局域网；

  - 服务器端口号未开放；

  - IP 地址填写错误。
解决方法：

  - 将手机热点或路由器设置为统一网络；

  - 使用网络调试助手开启 TCP 服务端并监听端口；

  - 在程序中重新设置正确的服务器 IP 与端口号，连接恢复正常。

- GPS 北斗数据解析异常，LCD 不显示定位信息
问题现象： 虽然能收到串口数据，但解析出的经纬度为空或全为 0。
原因分析：

  - 串口接收环形缓冲区指针未正确更新；

  - 多条 NMEA 语句混叠，未正确匹配 $GNGGA 字段。
解决方法：

  - 调整缓冲区指针更新逻辑，确保 RxDataOutPtr 与 RxDataInPtr 正确循环；

  - 在解析前先使用 memcmp 校验语句头；

  - 重新解析后 LCD 正常显示时间、经纬度和卫星数量。

- 液晶显示内容闪烁或覆盖不完全的问题
问题现象： 多次刷新显示内容时出现闪烁或旧内容残留。
原因分析： 每次显示内容时未清除原位置区域，导致字体重叠。
解决方法：

  - 在更新 GPS 信息前，对固定坐标区域使用背景色覆盖；

  - 或增加刷新间隔，避免频繁刷屏。

  - 修改后显示正常稳定。

六、实验思考与总结

USART通信至少需要两个引脚，一个接受数据输入RX引脚，一个发送数据输出TX引脚，在同步模式下还需要一个SLCK引脚以输出数据时钟，在硬件流模式下，还需一个nRTS引脚与nCTS引脚，n代表低电平有效，RTS为低时，代表USART准备好接受新数据，CTS为低时，代表可以发送数据。USART在发送数据时，发送数据由一位起始位、7或8位有效数据位、一位校验位以及停止位构成。，并且在连续发送数据时，只有在当前数据发送完毕后才可以进行下一次的数据传输，当USART_SR的TC位置为1时，发送结束。需要奇偶校验时，USART_CR1寄存器的M位需要设置为1，即9数据位。将USART_CR1寄存器的PCE位置1就可以启动奇偶校验控制。将USART_CR1寄存器的RE位置1，就可以使能USART接收。

附录：实验代码

1. 基础实验部分

（1）实验主要功能代码注释和分析

/****************************************Copyright (c)***************************************

**-------------------------------------------------------------------------------------------

** Created by:          qinyx

** Last modified Date:  2014-02-28

** Last Version:        V1.00

** Descriptions:        STM32F407嵌入式实验箱

**-------------------------------------------------------------------------------------------*/

//********************************************************************************************/

#include "stm32f4xx.h"

#include "USART.h"

#include "delay.h"

#include "LED.h"

int main(void)

{

SysTick_Init();                                                     //  系统滴答定时器初始化

UART3_Configuration();                                              //  USART3配置

printf("USART3 test\r\n");

printf("Input data:\r\n");

while (1)

{

if (Uart3.ReceiveFinish)

{

USART3_Senddata(Uart3.Rxbuf, Uart3.RXlenth);

Uart3.ReceiveFinish = 0;

Uart3.RXlenth = 0;

printf("\r\n");

}

}

}

2. 提高实验部分

基于WIFI的TCP服务端实验:

/****************************************Copyright (c)***************************************

**-------------------------------------------------------------------------------------------

** Created by:          qinyx

** Last modified Date:  2014-02-28

** Last Version:        V1.00

** Descriptions:        STM32F407嵌入式实验箱

**-------------------------------------------------------------------------------------------*/

//********************************************************************************************/

#include "stm32f4xx.h"

#include "USART3.h"

#include "delay.h"

#include "stdio.h"

#include "KEY.h"

#include "mp3.h"

#include "LED.h"

#include "WIFI.h"

#include "lcd.h"

int main(void)

{

unsigned char temp = 0;

unsigned char test[11] = "WIFI test\r\n";

SysTick_Init();                                                     //  系统滴答定时器初始化

LEDGpio_Init();

KEYGpio_Init();

UART3_Configuration();                                              //  USART3配置

UART5_Configuration(115200);

LCD_Init();                                                         //  液晶初始化

LCD_Clear(WHITE);                                                   //  设置液晶背景

LCD_ShowString(150, 50, "基于WIFI的TCP服务器实验", BLACK, WHITE);

LCD_ShowString(100, 100, "正在初始化wifi模块...", BLACK, WHITE);

printf("正在初始化wifi模块...\r\n");

delay_ms(500);

WIFI_Configuration();               //  wifi配置

LCD_ShowString(100, 100, "wifi模块初始化成功   ", BLACK, WHITE);

printf("wifi模块初始化成功   \r\n");

Uart5.ReceiveFinish = 0;

Uart5.RXlenth = 0;

Uart5.Time = 0;

while (1)

{

temp = KeyScan();

if (temp == 1)

{

WIFI_Send(test, sizeof(test));//  wifi发送数据

}

WIFI_Receive();                 //  wifi接收数据

}

}

基于WIFI的TCP客户端实验:

/****************************************Copyright (c)***************************************

**-------------------------------------------------------------------------------------------

** Created by:          qinyx

** Last modified Date:  2014-02-28

** Last Version:        V1.00

** Descriptions:        STM32F407嵌入式实验箱

**-------------------------------------------------------------------------------------------*/

//********************************************************************************************/

#include "stm32f4xx.h"

#include "USART3.h"

#include "delay.h"

#include "stdio.h"

#include "KEY.h"

#include "mp3.h"

#include "LED.h"

#include "WIFI.h"

#include "lcd.h"

int main(void)

{

unsigned char temp = 0;

unsigned char ON[2] = "ON";

unsigned char OFF[3] = "OFF";

SysTick_Init();                                                     //  系统滴答定时器初始化

LEDGpio_Init();

KEYGpio_Init();

UART3_Configuration();                                              //  USART3配置

//    UART6_Configuration(9600);

UART5_Configuration(115200);

LCD_Init();                                                         //  液晶初始化

LCD_Clear(WHITE);                                                   //  设置液晶背景

LCD_ShowString(150, 50, "基于WIFI的TCP客户端实验", BLACK, WHITE);

LCD_ShowString(100, 80, "模块初始化失败，尝试给模块重新上电", BLACK, WHITE);

LCD_ShowString(100, 100, "正在初始化wifi模块...", BLACK, WHITE);

printf("正在初始化wifi模块...\r\n");

delay_ms(500);

WIFI_Configuration();               //  wifi配置

LCD_ShowString(100, 100, "wifi模块初始化成功   ", BLACK, WHITE);

printf("wifi模块初始化成功   \r\n");

Uart5.ReceiveFinish = 0;

Uart5.RXlenth = 0;

Uart5.Time = 0;

while (1)

{

temp = KeyScan();

if (temp == 1)

{

WIFI_Send(ON, sizeof(ON));

}

else if (temp == 2)

{

WIFI_Send(OFF, sizeof(OFF));

}

WIFI_Receive();

}

}

GPS北斗实验:

/****************************************Copyright (c)***************************************

**-------------------------------------------------------------------------------------------

** Created by:          qinyx

** Last modified Date:  2014-02-28

** Last Version:        V1.00

** Descriptions:        STM32F407嵌入式实验箱

**-------------------------------------------------------------------------------------------*/

//********************************************************************************************/

#include "stm32f4xx.h"

#include "USART3.h"

#include "delay.h"

#include "stdio.h"

#include "KEY.h"

#include "mp3.h"

#include "LED.h"

#include "WIFI.h"

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <stdbool.h>

#include <math.h>

#include "GPS.h"

#include "lcd.h"

int main(void)

{

char dispbuf[100] = "";

SysTick_Init();                                                     //  系统滴答定时器初始化

LEDGpio_Init();

KEYGpio_Init();

UART3_Configuration();                                              //  USART3配置

GPS_Buff_Init();

UART5_Configuration(9600);

printf("GPS+北斗实验\r\n");

LCD_Init();                     //  液晶初始化

LCD_Clear(WHITE);               //  设置液晶背景

LCD_ShowString(180, 50, "GPS+北斗实验", BLACK, WHITE);

while (1)

{

if(RxDataOutPtr != RxDataInPtr)   //if成立的话，说明接收缓冲区有数据了

{

if (memcmp(&RxDataOutPtr[2], "$GNGGA", 6) == 0)

{

GNGGA_info(&RxDataOutPtr[2]);

printf("时间：%s\r\n", gps_time);

printf("经度：%6.2f%c\r\n", longitude,longitude_dir);

printf("纬度：%6.2f%c\r\n", latitude,latitude_dir);

sprintf(dispbuf, "时间：%s", gps_time);

LCD_ShowString(150, 200, (u8*)dispbuf,BLACK, WHITE);

sprintf(dispbuf, "经度：%6.2f%c", longitude,longitude_dir);

LCD_ShowString(150, 220, (u8*)dispbuf,BLACK, WHITE);

sprintf(dispbuf, "纬度：%6.2f%c", latitude,latitude_dir);

LCD_ShowString(150, 240, (u8*)dispbuf,BLACK, WHITE);

}

else if (memcmp(&RxDataOutPtr[2], "$GPGSV", 6) == 0)

{

GSV_info(&RxDataOutPtr[2]);

printf("可见GPS卫星总数:%d\r\n ",gpgsv.numSv);

sprintf(dispbuf, "可见GPS卫星总数:%d\r\n ",gpgsv.numSv);

LCD_ShowString(150, 300, (u8*)dispbuf,BLACK, WHITE);

}

else if (memcmp(&RxDataOutPtr[2], "$BDGSV", 6) == 0)

{

GSV_info(&RxDataOutPtr[2]);

printf("可见北斗卫星总数:%d\r\n ",gpgsv.numSv);

sprintf(dispbuf, "可见北斗卫星总数:%d ",gpgsv.numSv);

LCD_ShowString(150, 320, (u8*)dispbuf,BLACK, WHITE);

}

RxDataOutPtr += RBUFF_UNIT;                       //指针下移

if(RxDataOutPtr == RxDataEndPtr)                  //如果指针到缓冲区尾部了

RxDataOutPtr = RxDataBuf[0];                  //指针归位到缓冲区开头

}

}

}